6 CCQ-50 type double-pan twisted hair roasting drying machine

Preface

Thanks for you to select yiyuan brand products, in order to you can better use this product, produce more top tea, please be sure to read the instruction, in order to avoid has the mistakes. Especially the following the more must pay attention to:

1. This factory product distribution motivation have three-phase and single phase (380 V and 220 V), the user in the points when the choose and buy according to using places shall be determined, and power supply conditions Install the check to power and product motor is consistent.

2. For your safety, please be sure to pick up a grounded on machine shell, ground wire line can be fixed in the machine a any thread connection place, contact must be good.

3. Our factory DianReShi (known as burning electric) products use 220 V electric heat pipe, the power supply should be three-phase four wire, must have the zero line.

4. Our factory coal (coal or wood) products are used closed chamber and flue chimney height, by carefully research investigate sure. The chimney with random shortened or not should not arbitrarily, also do not optional another matching long for burning not free or heat escape too much tea quality, influence.

5. Gas type tea machine applied liquefied petroleum gas (usually about gas), if you use or natural gas pipeline gas, the fuel gas stove shall be change accordingly. Another gas stove should according to specifications choose corresponding pressure reducing valve, use high voltage, heat resistance, acid and alkali resistant tracheal connect the unit inlet interface, check the intake pipe burst, if there is any breakage replaced immediately, ensure security.

One, use and characteristics

This product is suitable for all kinds of curly type (including ball type) of excellent tea do form dry fry homework.

Different from common ball fry dry machine, this product has continuously variable institutions, don't stop the am institutions and strong hot air platoon is wet structure. Tea has made the bands tight knot, curly YunJi, colour and lustre is green, baekho evident, and the sweet thou, taste and gas, etc.

Two, the main technical parameters

| ITEM |  |  |  | Measurement unit | parameters |
| --- | --- | --- | --- | --- | --- |
| Fry pan | Diameter Ф x depth h |  |  | MM | Ф500×320 |
|  | quantity |  |  | individual | 2 |
| Leaf quantity for |  |  |  | KG | 8 to 12 (first baking leaves) |
| Fry board | Fry picture |  |  |  | 75-90 |
|  | Fry speed |  |  | R,P,M | 65-95 |
| Heat source type | electrothermic type |  | Not into the wind | KW,H | 6 |
|  |  |  | Strong into wind |  | 7.6 |
|  | Wood coal type |  |  | KG/H |  |
|  | Burning coals type |  |  |  | 0.6 kg of carbon / 1 kg dry tea |
|  | gas-powered |  |  | KG/H | 0.5 |
| electric motor |  |  |  | KW/V | 0.55/380(220) |
| Shape dimension (long x wide x high) |  | Not into the wind |  | MM | 1800×800×1000 |
|  |  | Strong into wind |  |  | 2100×800×1000 |
| Operating weight |  |  |  | KG | 200 |

Three, operation and maintenance

1, the preparation of the operation

(1) carefully check the power supply voltage and use the product, the voltage of power supply are in accord, and take the completed the ground, in order to ensure the safety of people and equipment.

(2) the machine should be placed in the ground level off, smooth and adjust; Should be cleaning up machine, especially within the fry pan inside and outside sundry, dirt, in order to protect the clean health certificate tea.

(3) switch on the power, start machine for half an hour, and carefully examine the idle running all moving parts is running no normal, and presence of abnormal sound and jam phenomenon, and the fill lube moving parts.

(4) check the fasteners is reliable, strong, screw loose fasteners.

2, operation

(1) the fuel gas type in the open gas ball valve total switch two seconds later, press the button, lit the ignition burner.In the ignition delay after angry, if not (or not lit burner completely lit) that close again, ball valves switch the ignition.

(2) heating up, when the temperature is up to reserve requirement of process, start motor, fry board, to cast leaves, multiple-refined treatments.and homework. When multiple-refined treatments.and, must certainly grasp the pot, MAO fire temperature (a small pot) pot in 90-110 temperature, every pot the steamed into two (as before baking 4 to 6 kg leaves), sufficient fire (pot) pot in 60-80 ℃ temperature, each leaf quantity pot shots of 4 to 6 kg (small pot leaves), to be the tea to process requirement multiple-refined treatments.and shape and dry degree, it can be.

(3), according to situation multiple-refined treatments and to fry, rotational speed adjustment of, such as wool fire (small pot) or at the beginning or the fry beginning in high temperature, at the same time, using a low speed, temperature multiple-refined treatments.and gear after to process requirement, temperature first after tea, with high low enough fire after the first water loss, low temperature, high low on low speed, small fry long multiple-refined treatments.and.

All in all, according to the quality of the tea around all kinds of curly of process operation for requirements and temperature, and shooting, leaf quantity to stay fry picture on speed,

reasonable adjustment.

(4) this product is open mouth, a spirited tea without a tea door. The tea, pull (the clutch handle or stop, stop check job with the hand, fry plate rotating fry board and reverse rotation from the tea and tea in the mouth.

(5) gas type in after use, first off, then switch knob, finally shut off the ball valve switch always open source shut.

3, maintenance

(1) before each use, shall be removed within and around the machine pot sundry (including broken tea), lest affect is often running and the tea quality, health.

(2) often pay attention to the work of the fastener tighten situation, found that the loose or fall off to handle in time.

(3) regularly on the transmission parts all lubricating points charging right amount of oil and adjust transmission v-belts tension degrees, make sure the machine working normally flexible.

(4) in the operation, such as the discovery of have abnormal sound, immediately stop check, find out the reason and remove the faults, can continue to work, it is strictly prohibited to run with faults.

(5) tea end of season, must carry on comprehensive maintenance, wear parts to replace, remove the pot to watch surface or add rust, and sealed with plastic cover protection, placed in dry ventilated place.

Four, common mechanical failure and elimination method (gas type) mainly related parts: О

| Failure or Associated parts | Point do not catch fire | Appear Yellow fire | Flame inequality | Burning noise | Maintenance method |
| --- | --- | --- | --- | --- | --- |
| Air source | O | O |  |  | Check whether angry, no gas should be Replacement air source |
| Burner |  | O | O | O | Clean burning fire hole |
| Electrical part | O |  |  |  | Adjust the position and send fire exchange battery, after regulation not angry,should repair and exchange |
| Switch part | O |  |  |  | Check whether there is a foreign body |
| Jet nozzle | O | O |  | O | Clean up the nozzle blockage |
| Pressure reducing valve | O |  |  |  | Pressure reducing valve and should be replaced |
| Pipeline |  |  |  | O | Check air tightness |
| Operating rules | O |  |  | O | According to the operation procedures execution |