1. the purpose and characteristics

This product is used in tea baking, Titian's equipment, especially to improve the aroma of the famous high-end tea has a significant effect.

This product uses the box-type structure, controllable intake and exhaust system. Hot air penetration tea, promotes the production of aromatic substances, maintaining color green tea, and improves efficiency. This product is especially suitable for low temperature Titian processes, such as the dried to 90% of tea into the drying oven to about 95% after the temperature about 50 ℃ long-insulation, so that the tea leaves inside aromatic substances diffuse into the surface, reaching tea Titian purposes. Because hot air circulating; aroma is not compromised when Titian, and thus particularly good aroma, can also be high temperature rapid Titian Titian attention into the machine should be 90% of the dried tea leaves.

This product can also be used for other food, medicine and other drying operations, the drying operation intake and exhaust ports should be opened to drain moisture.

2. the safety precautions and warnings

1, in strict accordance with operating instructions;

2, the rack must be grounded to prevent electric shock when used

3, maintenance, be sure to turn off the power;

4, Do not touch hot parts of the body, beware of burns.

3. The main technical parameters

| Parameter name |  | Units of measurement | 6CTH3.0 |  | 6CTH6.0 | 6CTH12.0 |
| --- | --- | --- | --- | --- | --- | --- |
| Effective Spreading area |  | ㎡ | 2.9 |  | 5.8 | 11.6 |
| Sheng leaf disc number |  | single | 8 |  | 16 | 32 |
| Production efficiency |  | kg/h | ≥25 |  | ≥45 | ≥90 |
| Fragrance ratio |  | % | ≥90 |  | ≥90 | ≥90 |
| Fragrance leaf moisture |  | % | 5～7 |  | 5～7 | 5～7 |
| Electric heater power |  | kW | 4.5 |  | 6 | 12 |
| Motor | Power | ## kW | 0.55 |  | 0.55 | 2×0.55 |
|  | Voltage | ## V | 220/380 |  |  |  |
| Dimensions (L × W × H) |  | mm*mm*mm | 1140×880×1300 | 1140×880×1980 |  | 2210×880×1980 |
| Weight |  | kg | 300 | 400 |  | 780 |

Note: In the system or the new tea leaf moisture content should not exceed 10%

4. the principle and structure overview

This product uses the box-type structure (see schematic), 6CTH3.0-and 6CTH6.0 structure the same principle, only in cabinet dwarf, leaf discs containing less, no scroll wheel. 6CTH12 type the two sets is 6CTH6.0 compound. By a fan mounted on the electric heater heated air is pumped into, the circulating air duct into the Sheng Ye plate, hot tea penetrate into the return air duct, and then heated by electric heaters after continued use. If the moisture content of tea large, open exhaust valve and humid air from the air intake goalkeeper.

6. installation and adjustment

Easy to install the machine in selected position on the machine can be put in place, but must ensure the following:

①. Installation recommended machine placed in a dry and ventilated place.
②. Surface must be level.

1、Installation and commissioning

(1) This product is a box structure, without foundation, can be placed in the famous tea processing process in place, requiring ground level, stable place does not swing open the door after the car should be connected with flat ground if the height difference, should seek adjusted to the contour.

(2) Connected to the supply. Power supply should be AC 50HZ, single power phase 220V, the power supply at the user shall install fuses, switches and electrical shock protection, and housing connected to the ground, in order to ensure safe use.

(3) Test run

a、Connect the power after commissioning. Fan switch open, view fan steering, such as turn right, stop after the three-phase line in the two lines can be connected interchangeably, single-phase motors need to improve access into the line.

b、Listen to the sound. Not too loud fan is normal even if uneven sound or abnormal sound check to shut down.

c、Heating switch is turned on, the thermostat temperature display on the instrument gradually increased, but the temperature rose slowly, and then heating the switch is open, the temperature rises rapidly, the current value increases, that is working properly.

d、Check the temperature control, time control is working properly. The temperature is set at a lower temperature, after a period of increased heating temperature can be set automatically stop heating, and the heat indicator is parked off to work; Set the time to a few minutes to reach the set time heating, plus Wen stopped off lights automatically stops and is working properly. When it reaches the set temperature but not to set the time, the system can automatically adjust the control to enter the state, ie the system reaches the set temperature automatically stop heating, heating continues, after cooling below the set temperature, automatic heating system and only set time is reached, the system stopped working (the fan stop).

e、The Sheng Ye Panqing clean, inside clean, empty plate heated to a temperature slightly above, and the inlet and exhaust doors open grill about 1 hour, drain the tank odor before official use.

2、Operating use

(1) Adjusted according to process requirements timing controller set time (for 6 hours 30 minutes, the timing controller of four squares can be adjusted to 0630), adjust the temperature controller set temperature, heating to set given temperature, and then filled with leaf disc tray rack pushed into the box, heating Titian operations. During the operation, inlet exhaust doors can be adjusted inside the water vapor content, to ensure green tea.

(2) Tea shop on the Sheng Ye disk requires uniform thickness appropriate adjustments according to process requirements.

(3) Add tea leaves, close the door, and then start the fan, open after heating, heating switch, temperature and time settings into the automatic control state, the system automatically after a set time to stop working (fan stop), restart the system for, you must first turn off the fan switch, then start the fan switch to Titian job is complete, you can remove the filled leaf discs. Turn off heat, heating switch, 10 minute (electric heater cooling) before they can turn off the fan switch.

(4) 6CTH6.0 and 12.0 Titian machine type of tea: tea tray rack placed on a trolley, and the car frame with pin connection, installed onto Titian after tea machine door, trolley on rails and boxes on rails, remove push the latch inside the frame. When the tea tray with a dedicated rack pull hook will pull carts, plug connection bolt, move additionally put cooling, such as the user subscription sets Sheng Ye dish, tray racks, carts, you can Titian continuous operation.

6CTH3.0 type of tea Titian machine: the need to machine directly from Titian Titian's tea tray has been removed, replaced by disk into tea and then, before proceeding Titian operations.

(5)Turn off the power after the fragrance is completed.

5 Maintenance

(1) Motor temperature rise should not exceed 65 ℃.

(2) Note that when using electric heating meter readings, such as current indicates that the value becomes smaller, it means that there is an electric wire blown, should be shut down after cooling overhaul.

(3) Power, fan switch on, switch on heating, heating switch on, the corresponding signal lights (power light, fan light, heating lamp, heat lamps) should be lit, where there is not bright phenomenon should stop check.

(4) Electrical malfunction, you should consult an electrician for repair.

6.cause and remedy

| No. | Fault phenomenon | Analysis of the reasons | clusion method |
| --- | --- | --- | --- |
| 1 | Fan does not move, there is buzzing sound | The motor is stuck | Check motor fan, the fan Troubleshooting |
| 2 | Hemp hand body has a sense of shock | Ungrounded or poorly grounded enclosure, leakage | Take a good ground, check whether the leakage electric wire or wires |
| 3 | Thermostat not working | Thermostat damage | replace |
| 4 | Time control allowed | Timer switch is damaged | replace |
| 5 | Work during a sudden stop | Motor overheating | After cooling down to start or transfer Festival thermal relay capacity |

7. Other

1、Shipping list

| Name | 6CTH3.0 | 6CTH6.0 | 6CTH12.0 |
| --- | --- | --- | --- |
| Host machine | 1 | 1 | 1 |
| Push car | 0 | 1 | 2 |
| Tea shelf | 1(625*664*587) | 1(625*664*1226) | 2(625*664*1226) |
| Sheng tea drawer (On the mainframe) | 8 | 16 | 32 |
| Pin plug | 0 | 1 | 2 |
| Pull hook | 0 | 1 | 1 |
| Instructions | 1 | 1 | 1 |
| Certificate | 1 | 1 | 1 |
| Warranty | 1 | 1 | 1 |

For users increased carts, tea stand, Sheng tea drawer, to be stated in the contract, and a separate valuation.