### Floor scale Operating Manual

1. Specification

| Loaded weight of load cell | Platform size （m×m） | Platform height（cm） | explanation |
| --- | --- | --- | --- |
| 1t | 1×1 | 11～17cm selectable | The trim size can make according to customer’s request |
| 2t | 1.2×1.2 |  |  |
| 3t | 1.5×1.5 |  |  |

2. installation

1). Remove the package, use the four bottom screw in the parts to install in the four feet of the platform

2). Place the scale to the level off ground, adjust the four bottom screw to level

3). Connect the floor scale with the indicator (XK3100-A1 or other indicator ) and fixed up

3. interface

The four shield line of floor scale use 9 pin plug, lead foot define as follows:

o  o  o  o  o

o  o  o  o

1|  2|  3|  4|  5|

6|  7|  8|  9|

PIN1、PIN2: Load cell excitation voltage（＋）

PIN4、PIN5: Load cell excitation voltage（-）

PIN6、PIN7:load cell input ＋

PIN8、PIN9:load cell output －

PIN3:empty